WEBMART

WiFi Captive Portal System

Full Implementation & Deployment Guide

| Version | 1.0 |
| --- | --- |
| Platform | Proxmox LXC / Debian 12 |
| Stack | Django · PostgreSQL · hostapd · dnsmasq · nftables |

# 1. System Overview & Architecture

Webmart is a WiFi captive portal that intercepts guest devices connecting to the Webmart Guest hotspot. Upon connection, a Django-backed web application is served that handles user registration, token-based access management, and provides authenticated access to home media services such as Jellyfin and Navidrome.

## 1.1 Architecture Diagram

Component Flow: Device → AP (hostapd) → dnsmasq DHCP → iptables redirect → Django Portal → Token Validation → Media Services

| Layer | Component | Technology | Role |
| --- | --- | --- | --- |
| Access Point | Wireless AP | hostapd + Atheros USB | Creates Webmart Guest SSID |
| DHCP/DNS | Network Services | dnsmasq | IP assignment, DNS captive |
| Firewall | Traffic Control | nftables | Redirect HTTP to portal |
| Backend | Web Application | Django 4.x + PostgreSQL | Auth, tokens, dashboard |
| Reverse Proxy | HTTP Server | Nginx | TLS, static files |
| Media | Content Servers | Jellyfin / Navidrome | Protected media access |
| Container | Isolation | Proxmox LXC (Debian 12) | Runs entire stack |

## 1.2 Key Features

- User self-registration (email, phone, date of birth)

- Automatic award of 5 access tokens on registration

- Each token grants 5 hours of media server access

- Token purchase system for additional tokens

- MAC address and device-type fingerprinting bound to user profile

- Real-time bandwidth and session-time tracking per user in admin

- Admin dashboard with full user analytics

- Atheros USB dongle managed as dedicated AP interface

# 2. Prerequisites

## 2.1 Hardware Requirements

| Component | Specification | Notes |
| --- | --- | --- |
| Proxmox Host | Any x86_64 CPU, 4+ GB RAM | Runs the LXC container |
| USB WiFi Dongle | Atheros AR9271 or similar | Must support AP mode |
| Storage | 20+ GB for container | SSD recommended |
| LAN | Ethernet to router/internet | For media server access |

## 2.2 Software Prerequisites on Proxmox Host

- Proxmox VE 7.x or 8.x installed

- Debian 12 (Bookworm) LXC template downloaded

- USB dongle passed through to container (privileged LXC or USB passthrough)

- Internet access on Proxmox host for package downloads

WARNING: The LXC container must be created as privileged, or the USB device must be explicitly passed through, for hostapd to control the wireless interface.

# 3. Proxmox LXC Container Setup

## 3.1 Create the Container

On the Proxmox host shell, execute the following steps to create and configure the Debian container:

### Step 1 — Download the Debian 12 template

# On the Proxmox host shell

pveam update

pveam available | grep debian-12

pveam download local debian-12-standard_12.7-1_amd64.tar.zst

### Step 2 — Identify your USB dongle

# Plug in the Atheros USB dongle, then run:

lsusb

# Look for: Atheros Communications, Inc. AR9271 802.11n

# Note the Bus and Device numbers e.g.: Bus 002 Device 003

# Also note the device path:

ls /dev/bus/usb/002/

### Step 3 — Create the LXC container (privileged)

# Create a privileged container — required for hostapd / raw net access

pct create 200 local:vztmpl/debian-12-standard_12.7-1_amd64.tar.zst \

--hostname webmart \

--cores 2 \

--memory 2048 \

--swap 512 \

--storage local-lvm \

--rootfs local-lvm:20 \

--net0 name=eth0,bridge=vmbr0,ip=dhcp \

--unprivileged 0 \

--features nesting=1

### Step 4 — Pass through the USB WiFi adapter

# Add USB device passthrough to container config

# Replace 2-3 with your actual Bus-Device numbers

echo 'usb0: host=2-3' >> /etc/pve/lxc/200.conf

# Also allow raw network device access in container

echo 'lxc.cgroup2.devices.allow: c 189:* rwm' >> /etc/pve/lxc/200.conf

echo 'lxc.mount.entry: /dev/bus/usb dev/bus/usb none bind,optional,create=dir 0 0' >> /etc/pve/lxc/200.conf

### Step 5 — Start the container and enter it

pct start 200

pct enter 200

# You are now inside the Debian 12 container

# 4. Base System Configuration Inside the Container

## 4.1 Update & Install Core Packages

apt update && apt upgrade -y

apt install -y \

hostapd dnsmasq nftables \

python3 python3-pip python3-venv \

postgresql postgresql-contrib \

nginx \

git curl wget usbutils iw wireless-tools \

net-tools iproute2 \

redis-server \

supervisor

## 4.2 Identify the Wireless Interface

# Check that the USB dongle is visible

lsusb

# Should show: Atheros AR9271

# Find the wireless interface name

iw dev

# Typically: wlan0 or wlx<macaddress>

# Confirm it supports AP mode

iw phy phy0 info | grep -A 10 'Supported interface modes'

# Must show: AP in the list

NOTE: Throughout this guide, the wireless interface is referred to as wlan0. Replace with your actual interface name if different (e.g. wlx00e04c360001).

## 4.3 Disable NetworkManager / systemd-networkd for wlan0

# Prevent other services from managing wlan0

systemctl disable --now wpa_supplicant 2>/dev/null || true

# Create a static config to prevent DHCP on wlan0

cat > /etc/network/interfaces.d/wlan0 << 'EOF'

auto wlan0

iface wlan0 inet static

address 192.168.10.1

netmask 255.255.255.0

EOF

# Bring up wlan0 with static IP

ip addr add 192.168.10.1/24 dev wlan0 2>/dev/null || true

ip link set wlan0 up

# 5. Access Point Configuration — hostapd

## 5.1 Create the hostapd Configuration File

cat > /etc/hostapd/hostapd.conf << 'EOF'

# Interface & driver

interface=wlan0

driver=nl80211

# SSID

ssid=Webmart Guest

# Frequency & channel

hw_mode=g

channel=6

ieee80211n=1

wmm_enabled=1

# Security — open network for captive portal

auth_algs=1

wpa=0

# Country code — adjust to your region

country_code=ZW

ieee80211d=1

# MAC address control (allow all)

macaddr_acl=0

# Logging

logger_syslog=-1

logger_syslog_level=2

logger_stdout=-1

logger_stdout_level=2

EOF

## 5.2 Point systemd at the Config File

sed -i 's|#DAEMON_CONF=""|DAEMON_CONF="/etc/hostapd/hostapd.conf"|' /etc/default/hostapd

## 5.3 Enable and Start hostapd

systemctl unmask hostapd

systemctl enable hostapd

systemctl start hostapd

systemctl status hostapd

# Verify the SSID is broadcasting:

iw dev wlan0 info

# 6. DHCP & DNS — dnsmasq

dnsmasq serves IP addresses to connecting clients and resolves all DNS queries to the portal IP — the core mechanism of the captive portal redirect.

## 6.1 Configure dnsmasq

cat > /etc/dnsmasq.conf << 'EOF'

# Interface to serve

interface=wlan0

bind-interfaces

# DHCP pool: 192.168.10.50 – 192.168.10.200

dhcp-range=192.168.10.50,192.168.10.200,255.255.255.0,12h

# Gateway and DNS for clients

dhcp-option=3,192.168.10.1

dhcp-option=6,192.168.10.1

# Captive portal: resolve ALL domains to portal IP

# This triggers the 'Sign in to network' prompt on devices

address=/#/192.168.10.1

# Except these — resolve normally for media servers

# (Uncomment and adjust after media servers are set up)

#server=/jellyfin.local/192.168.1.x

#server=/navidrome.local/192.168.1.x

# Log DHCP assignments (for MAC binding)

log-dhcp

log-facility=/var/log/dnsmasq.log

# Store DHCP leases

dhcp-leasefile=/var/lib/misc/dnsmasq.leases

# Captive portal detection responses

# These special domains are probed by OS captive portal detection

address=/connectivitycheck.gstatic.com/192.168.10.1

address=/www.msftconnecttest.com/192.168.10.1

address=/captive.apple.com/192.168.10.1

address=/detectportal.firefox.com/192.168.10.1

EOF

## 6.2 Enable dnsmasq

systemctl enable dnsmasq

systemctl restart dnsmasq

systemctl status dnsmasq

# 7. Firewall & Traffic Routing — nftables

nftables controls three things: (1) NAT for client internet access after authentication, (2) HTTP/HTTPS redirect to the captive portal for unauthenticated clients, and (3) bandwidth accounting per IP address.

## 7.1 IP Forwarding

# Enable IP forwarding permanently

echo 'net.ipv4.ip_forward=1' >> /etc/sysctl.conf

sysctl -p

## 7.2 Create the nftables Ruleset

cat > /etc/nftables.conf << 'NFTEOF'

#!/usr/sbin/nft -f

flush ruleset

# ── Variables ──────────────────────────────────────────────────────

define WAN_IF  = eth0          # upstream interface (internet)

define WLAN_IF = wlan0         # AP interface (guests)

define PORTAL  = 192.168.10.1  # portal IP

define PORTAL_PORT = 80        # Django/Nginx listen port

# ── NAT table ───────────────────────────────────────────────────────

table ip nat {

# Authenticated clients set — managed by Django via nft command

set allowed_ips {

type ipv4_addr

flags timeout

comment "IPs of authenticated/token-active clients"

}

chain PREROUTING {

type nat hook prerouting priority dstnat; policy accept;

# Pass through authenticated clients

ip saddr @allowed_ips accept

# Redirect all HTTP from unauthenticated clients to portal

iif $WLAN_IF tcp dport 80  redirect to :$PORTAL_PORT

# Redirect HTTPS to portal (will show cert warning — expected)

iif $WLAN_IF tcp dport 443 redirect to :$PORTAL_PORT

}

chain POSTROUTING {

type nat hook postrouting priority srcnat; policy accept;

# Masquerade authenticated clients to WAN

ip saddr @allowed_ips oif $WAN_IF masquerade

}

}

# ── Filter table ────────────────────────────────────────────────────

table ip filter {

set allowed_ips {

type ipv4_addr

flags timeout

comment "Mirror of nat::allowed_ips for forward filtering"

}

chain FORWARD {

type filter hook forward priority filter; policy drop;

# Allow established/related traffic

ct state established,related accept

# Allow authenticated clients to reach WAN

ip saddr @allowed_ips oif $WAN_IF accept

ip daddr @allowed_ips iif $WAN_IF accept

}

chain INPUT {

type filter hook input priority filter; policy drop;

ct state established,related accept

# Allow DHCP from clients

iif $WLAN_IF udp dport 67 accept

# Allow DNS from clients

iif $WLAN_IF tcp dport 53 accept

iif $WLAN_IF udp dport 53 accept

# Allow HTTP/HTTPS to portal

iif $WLAN_IF tcp dport { 80, 443 } accept

# Allow loopback

iif lo accept

# Allow SSH from LAN (management)

iif eth0 tcp dport 22 accept

}

}

# ── Accounting table ─────────────────────────────────────────────────

# Per-IP byte counters, reset hourly by Django management command

table ip accounting {

chain rx {

type filter hook input priority 100; policy accept;

iif $WLAN_IF counter

}

chain tx {

type filter hook output priority 100; policy accept;

oif $WLAN_IF counter

}

}

NFTEOF

## 7.3 Apply and Enable nftables

nft -f /etc/nftables.conf

systemctl enable nftables

systemctl start nftables

# Verify the ruleset loaded correctly

nft list ruleset

## 7.4 Adding / Removing Authenticated IPs from Django

Django will call these commands when a token is activated or expires:

# Add an authenticated IP (with 5-hour timeout = 18000 seconds)

nft add element ip nat allowed_ips { 192.168.10.55 timeout 18000s }

nft add element ip filter allowed_ips { 192.168.10.55 timeout 18000s }

# Remove an IP immediately (on logout or token expiry)

nft delete element ip nat allowed_ips { 192.168.10.55 }

nft delete element ip filter allowed_ips { 192.168.10.55 }

# 8. Database Setup — PostgreSQL

# Start PostgreSQL

systemctl enable postgresql

systemctl start postgresql

# Create database and user

sudo -u postgres psql << 'SQL'

CREATE USER webmart_user WITH PASSWORD 'CHANGE_THIS_STRONG_PASSWORD';

CREATE DATABASE webmart_db OWNER webmart_user;

GRANT ALL PRIVILEGES ON DATABASE webmart_db TO webmart_user;

\q

SQL

# 9. Django Application Setup

## 9.1 Create Application Directory & Virtual Environment

mkdir -p /opt/webmart

cd /opt/webmart

python3 -m venv venv

source venv/bin/activate

pip install --upgrade pip

pip install django psycopg2-binary djangorestframework \

django-crispy-forms crispy-bootstrap5 \

pillow python-decouple celery redis \

django-allauth whitenoise gunicorn

## 9.2 Create the Django Project & Apps

cd /opt/webmart

source venv/bin/activate

django-admin startproject config .

# Create the portal apps

python manage.py startapp portal     # user-facing portal

python manage.py startapp accounts   # registration & auth

python manage.py startapp tokens     # token management

python manage.py startapp devices    # MAC/device tracking

python manage.py startapp analytics  # bandwidth & time tracking

## 9.3 Environment Variables (.env)

cat > /opt/webmart/.env << 'EOF'

SECRET_KEY=replace-with-django-secret-key-50-chars

DEBUG=False

ALLOWED_HOSTS=192.168.10.1,webmart.local,localhost

DB_NAME=webmart_db

DB_USER=webmart_user

DB_PASSWORD=CHANGE_THIS_STRONG_PASSWORD

DB_HOST=localhost

DB_PORT=5432

REDIS_URL=redis://localhost:6379/0

# Registration gift tokens

REGISTRATION_TOKEN_GRANT=5

TOKEN_DURATION_HOURS=5

EOF

## 9.4 Django Settings (config/settings.py)

# config/settings.py  — key sections only

from decouple import config

import os

from pathlib import Path

BASE_DIR = Path(__file__).resolve().parent.parent

SECRET_KEY = config('SECRET_KEY')

DEBUG = config('DEBUG', cast=bool, default=False)

ALLOWED_HOSTS = config('ALLOWED_HOSTS').split(',')

INSTALLED_APPS = [

'django.contrib.admin',

'django.contrib.auth',

'django.contrib.contenttypes',

'django.contrib.sessions',

'django.contrib.messages',

'django.contrib.staticfiles',

# Third-party

'crispy_forms',

'crispy_bootstrap5',

# Webmart apps

'portal',

'accounts',

'tokens',

'devices',

'analytics',

]

DATABASES = {

'default': {

'ENGINE': 'django.db.backends.postgresql',

'NAME': config('DB_NAME'),

'USER': config('DB_USER'),

'PASSWORD': config('DB_PASSWORD'),

'HOST': config('DB_HOST', default='localhost'),

'PORT': config('DB_PORT', default='5432'),

}

}

STATIC_URL = '/static/'

STATIC_ROOT = BASE_DIR / 'staticfiles'

MEDIA_URL = '/media/'

MEDIA_ROOT = BASE_DIR / 'media'

CRISPY_ALLOWED_TEMPLATE_PACKS = 'bootstrap5'

CRISPY_TEMPLATE_PACK = 'bootstrap5'

LOGIN_URL = '/accounts/login/'

LOGIN_REDIRECT_URL = '/dashboard/'

# Celery for async token expiry

CELERY_BROKER_URL = config('REDIS_URL')

CELERY_RESULT_BACKEND = config('REDIS_URL')

# Custom settings

REGISTRATION_TOKEN_GRANT = config('REGISTRATION_TOKEN_GRANT', cast=int, default=5)

TOKEN_DURATION_HOURS = config('TOKEN_DURATION_HOURS', cast=int, default=5)

# 10. Django Data Models

## 10.1 User Profile — accounts/models.py

from django.db import models

from django.contrib.auth.models import AbstractUser

class WebmartUser(AbstractUser):

"""Extended user with portal-specific fields."""

email       = models.EmailField(unique=True)

phone       = models.CharField(max_length=20, blank=True)

date_of_birth = models.DateField(null=True, blank=True)

created_at  = models.DateTimeField(auto_now_add=True)

# Track tokens at a glance

@property

def available_tokens(self):

return self.token_set.filter(used=False, expired=False).count()

def __str__(self):

return f"{self.email} ({self.username})"

# Add to config/settings.py:

# AUTH_USER_MODEL = 'accounts.WebmartUser'

## 10.2 Token Model — tokens/models.py

import uuid

from django.db import models

from django.conf import settings

from django.utils import timezone

from datetime import timedelta

class Token(models.Model):

"""A single access token granting TOKEN_DURATION_HOURS of portal access."""

id         = models.UUIDField(primary_key=True, default=uuid.uuid4, editable=False)

user       = models.ForeignKey(settings.AUTH_USER_MODEL, on_delete=models.CASCADE)

created_at = models.DateTimeField(auto_now_add=True)

activated_at = models.DateTimeField(null=True, blank=True)

expires_at = models.DateTimeField(null=True, blank=True)

used       = models.BooleanField(default=False)

expired    = models.BooleanField(default=False)

source     = models.CharField(max_length=30, choices=[

('registration','Registration Gift'),

('purchase','Purchase'),

], default='registration')

def activate(self, ip_address):

"""Mark token active, set expiry, whitelist IP in nftables."""

self.used        = True

self.activated_at = timezone.now()

self.expires_at  = timezone.now() + timedelta(

hours=settings.TOKEN_DURATION_HOURS

)

self.save()

# Open firewall for this IP

import subprocess

seconds = settings.TOKEN_DURATION_HOURS * 3600

subprocess.run([

'nft', 'add', 'element', 'ip', 'nat', 'allowed_ips',

f'{{ {ip_address} timeout {seconds}s }}'

], check=False)

subprocess.run([

'nft', 'add', 'element', 'ip', 'filter', 'allowed_ips',

f'{{ {ip_address} timeout {seconds}s }}'

], check=False)

@property

def is_active(self):

if not self.used or self.expired:

return False

return timezone.now() < self.expires_at

def __str__(self):

return f'Token {str(self.id)[:8]} — {self.user.email}'

## 10.3 Device Model — devices/models.py

from django.db import models

from django.conf import settings

class Device(models.Model):

"""WiFi device fingerprint bound to a user profile."""

user        = models.ForeignKey(

settings.AUTH_USER_MODEL,

on_delete=models.SET_NULL,

null=True, blank=True,

related_name='devices'

)

mac_address = models.CharField(max_length=17, unique=True)

ip_address  = models.GenericIPAddressField(null=True, blank=True)

hostname    = models.CharField(max_length=255, blank=True)

device_type = models.CharField(max_length=100, blank=True)

os_hint     = models.CharField(max_length=100, blank=True)

user_agent  = models.TextField(blank=True)

first_seen  = models.DateTimeField(auto_now_add=True)

last_seen   = models.DateTimeField(auto_now=True)

def __str__(self):

return f'{self.mac_address} ({self.device_type or "Unknown"}) — {self.user}'

## 10.4 Analytics / Session Model — analytics/models.py

from django.db import models

from django.conf import settings

class Session(models.Model):

"""Tracks time and bandwidth for one portal session."""

user          = models.ForeignKey(settings.AUTH_USER_MODEL, on_delete=models.CASCADE)

device        = models.ForeignKey('devices.Device', on_delete=models.SET_NULL, null=True)

token         = models.OneToOneField('tokens.Token', on_delete=models.SET_NULL, null=True)

start_time    = models.DateTimeField(auto_now_add=True)

end_time      = models.DateTimeField(null=True, blank=True)

bytes_rx      = models.BigIntegerField(default=0)   # bytes downloaded by client

bytes_tx      = models.BigIntegerField(default=0)   # bytes uploaded by client

ip_address    = models.GenericIPAddressField()

@property

def duration_seconds(self):

if self.end_time:

return (self.end_time - self.start_time).seconds

from django.utils import timezone

return (timezone.now() - self.start_time).seconds

@property

def total_bytes(self):

return self.bytes_rx + self.bytes_tx

def __str__(self):

return f'Session {self.id} — {self.user.email}'

# 11. Key Django Views & Logic

## 11.1 MAC Address Detection Middleware — devices/middleware.py

Every request from the portal network is parsed for MAC address, device type, and OS hint.

import re

import subprocess

from .models import Device

def get_mac_from_arp(ip):

"""Look up MAC address from the kernel ARP table for the given IP."""

try:

result = subprocess.run(['arp', '-n', ip], capture_output=True, text=True)

match  = re.search(r'([0-9a-fA-F:]{17})', result.stdout)

return match.group(1).lower() if match else None

except Exception:

return None

def detect_device_type(user_agent):

ua = user_agent.lower()

if 'android' in ua:  return 'Android Phone/Tablet'

if 'iphone' in ua:   return 'iPhone'

if 'ipad' in ua:     return 'iPad'

if 'windows' in ua:  return 'Windows PC'

if 'macintosh' in ua or 'mac os x' in ua: return 'Mac'

if 'linux' in ua:    return 'Linux Device'

return 'Unknown'

class DeviceFingerprintMiddleware:

def __init__(self, get_response):

self.get_response = get_response

def __call__(self, request):

ip  = request.META.get('REMOTE_ADDR')

ua  = request.META.get('HTTP_USER_AGENT', '')

mac = get_mac_from_arp(ip)

if mac:

device, _ = Device.objects.update_or_create(

mac_address=mac,

defaults={

'ip_address': ip,

'user_agent': ua,

'device_type': detect_device_type(ua),

'user': request.user if request.user.is_authenticated else None,

}

)

request.device = device

else:

request.device = None

response = self.get_response(request)

return response

## 11.2 Registration View — accounts/views.py

from django.shortcuts import render, redirect

from django.contrib.auth import login

from django.conf import settings

from .forms import RegistrationForm

from tokens.models import Token

def register(request):

if request.method == 'POST':

form = RegistrationForm(request.POST)

if form.is_valid():

user = form.save()

# Grant registration tokens

for _ in range(settings.REGISTRATION_TOKEN_GRANT):

Token.objects.create(user=user, source='registration')

login(request, user)

return redirect('dashboard')

else:

form = RegistrationForm()

return render(request, 'accounts/register.html', {'form': form})

## 11.3 Dashboard & Token Activation — portal/views.py

from django.contrib.auth.decorators import login_required

from django.shortcuts import render, get_object_or_404, redirect

from tokens.models import Token

from analytics.models import Session

@login_required

def dashboard(request):

tokens   = Token.objects.filter(user=request.user).order_by('-created_at')

sessions = Session.objects.filter(user=request.user).order_by('-start_time')[:20]

return render(request, 'portal/dashboard.html', {

'tokens': tokens,

'sessions': sessions,

'available': tokens.filter(used=False, expired=False).count()

})

@login_required

def activate_token(request, token_id):

token = get_object_or_404(Token, id=token_id, user=request.user, used=False)

ip    = request.META.get('REMOTE_ADDR')

token.activate(ip)

# Create analytics session

Session.objects.create(

user=request.user,

device=getattr(request, 'device', None),

token=token,

ip_address=ip

)

return redirect('dashboard')

## 11.4 Bandwidth Polling — analytics/management/commands/poll_bandwidth.py

# Run via cron every 5 minutes: */5 * * * * /opt/webmart/venv/bin/python

#   /opt/webmart/manage.py poll_bandwidth

import subprocess, re

from django.core.management.base import BaseCommand

from analytics.models import Session

from django.utils import timezone

class Command(BaseCommand):

help = 'Poll nftables counters and update session bandwidth'

def handle(self, *args, **kwargs):

# Retrieve all active sessions

active = Session.objects.filter(end_time__isnull=True)

for session in active:

ip = session.ip_address

# Parse nftables counters for this IP

rx = self._get_bytes('input',  ip)

tx = self._get_bytes('output', ip)

session.bytes_rx = rx

session.bytes_tx = tx

# Close expired sessions

if session.token and not session.token.is_active:

session.end_time = timezone.now()

session.save()

def _get_bytes(self, direction, ip):

try:

result = subprocess.run(

['nft', '-j', 'list', 'ruleset'],

capture_output=True, text=True

)

# Parse JSON output for counter matching IP

# (Simplified — production should use nft -j and parse properly)

return 0  # placeholder

except Exception:

return 0

# 12. URL Configuration

# config/urls.py

from django.contrib import admin

from django.urls import path, include

urlpatterns = [

path('admin/',      admin.site.urls),

path('accounts/',   include('accounts.urls')),

path('tokens/',     include('tokens.urls')),

path('dashboard/',  include('portal.urls')),

path('',            include('portal.urls')),

]

# accounts/urls.py

from django.urls import path

from django.contrib.auth import views as auth_views

from . import views

urlpatterns = [

path('register/', views.register,         name='register'),

path('login/',    auth_views.LoginView.as_view(template_name='accounts/login.html'), name='login'),

path('logout/',   auth_views.LogoutView.as_view(next_page='/'), name='logout'),

]

# tokens/urls.py

from django.urls import path

from . import views

urlpatterns = [

path('activate/<uuid:token_id>/', views.activate_token, name='activate_token'),

path('buy/',                      views.buy_tokens,      name='buy_tokens'),

]

# 13. Admin Dashboard Configuration

## 13.1 Register Models with Enhanced Admin — accounts/admin.py

from django.contrib import admin

from django.contrib.auth.admin import UserAdmin

from django.utils.html import format_html

from .models import WebmartUser

from devices.models import Device

from tokens.models import Token

from analytics.models import Session

class DeviceInline(admin.TabularInline):

model   = Device

fk_name = 'user'

extra   = 0

readonly_fields = ('mac_address','ip_address','device_type','os_hint','first_seen','last_seen')

class TokenInline(admin.TabularInline):

model   = Token

extra   = 0

readonly_fields = ('id','source','used','expired','activated_at','expires_at')

class SessionInline(admin.TabularInline):

model   = Session

extra   = 0

readonly_fields = ('start_time','end_time','ip_address','bytes_rx','bytes_tx')

def bandwidth_display(self, obj):

mb = obj.total_bytes / (1024*1024)

return f'{mb:.2f} MB'

bandwidth_display.short_description = 'Total Bandwidth'

@admin.register(WebmartUser)

class WebmartUserAdmin(UserAdmin):

inlines      = [DeviceInline, TokenInline, SessionInline]

list_display = ('email','username','phone','available_tokens','total_sessions','date_joined')

search_fields = ('email','username','phone')

list_filter  = ('is_staff','is_active','date_joined')

def available_tokens(self, obj):

return obj.available_tokens

available_tokens.short_description = 'Tokens Left'

def total_sessions(self, obj):

return Session.objects.filter(user=obj).count()

total_sessions.short_description = 'Sessions'

@admin.register(Device)

class DeviceAdmin(admin.ModelAdmin):

list_display = ('mac_address','user','device_type','ip_address','first_seen','last_seen')

search_fields = ('mac_address','user__email','device_type')

list_filter  = ('device_type',)

@admin.register(Session)

class SessionAdmin(admin.ModelAdmin):

list_display = ('user','device','ip_address','start_time','duration','bandwidth')

readonly_fields = ('bytes_rx','bytes_tx')

def duration(self, obj):

d = obj.duration_seconds

return f'{d//3600}h {(d%3600)//60}m'

duration.short_description = 'Time Spent'

def bandwidth(self, obj):

mb = obj.total_bytes / (1024*1024)

return f'{mb:.2f} MB'

bandwidth.short_description = 'Bandwidth'

# 14. Nginx Reverse Proxy Configuration

cat > /etc/nginx/sites-available/webmart << 'EOF'

server {

listen 80 default_server;

server_name 192.168.10.1 webmart.local _;

# Captive portal detection endpoints

location /generate_204 {

return 302 http://192.168.10.1/;

}

location /hotspot-detect.html {

return 302 http://192.168.10.1/;

}

location /ncsi.txt {

return 302 http://192.168.10.1/;

}

# Static files served directly by Nginx

location /static/ {

alias /opt/webmart/staticfiles/;

expires 7d;

add_header Cache-Control public;

}

# Proxy everything else to Gunicorn

location / {

proxy_pass         http://127.0.0.1:8000;

proxy_set_header   Host             $host;

proxy_set_header   X-Real-IP        $remote_addr;

proxy_set_header   X-Forwarded-For  $proxy_add_x_forwarded_for;

proxy_read_timeout 60s;

}

}

EOF

# Enable the site

ln -sf /etc/nginx/sites-available/webmart /etc/nginx/sites-enabled/

rm -f /etc/nginx/sites-enabled/default

nginx -t && systemctl restart nginx

systemctl enable nginx

# 15. Gunicorn Process Manager — Supervisor

cat > /etc/supervisor/conf.d/webmart.conf << 'EOF'

[program:webmart_gunicorn]

command=/opt/webmart/venv/bin/gunicorn config.wsgi:application

--bind 127.0.0.1:8000

--workers 3

--timeout 120

--access-logfile /var/log/webmart/gunicorn_access.log

--error-logfile  /var/log/webmart/gunicorn_error.log

directory=/opt/webmart

user=www-data

autostart=true

autorestart=true

environment=DJANGO_SETTINGS_MODULE="config.settings"

[program:webmart_celery]

command=/opt/webmart/venv/bin/celery -A config worker --loglevel=info

directory=/opt/webmart

user=www-data

autostart=true

autorestart=true

EOF

mkdir -p /var/log/webmart

chown www-data:www-data /var/log/webmart

chown -R www-data:www-data /opt/webmart

systemctl enable supervisor

systemctl restart supervisor

supervisorctl status

# 16. Finalise Django & Run Migrations

cd /opt/webmart

source venv/bin/activate

# Point Django at custom user model in settings.py:

# AUTH_USER_MODEL = 'accounts.WebmartUser'

python manage.py makemigrations accounts devices tokens analytics

python manage.py migrate

# Collect static files

python manage.py collectstatic --noinput

# Create the superuser for admin access

python manage.py createsuperuser

# 17. Token Purchase System

Webmart supports token purchasing via manual payment (e.g., cash-based or local mobile money). The following outlines the backend structure; a payment gateway integration (Paynow, EcoCash, or Stripe) can be added as an extension.

## 17.1 Purchase Model — tokens/models.py (addition)

class TokenPurchase(models.Model):

"""Records a token purchase transaction."""

PENDING   = 'pending'

COMPLETED = 'completed'

FAILED    = 'failed'

STATUS_CHOICES = [(PENDING,'Pending'),(COMPLETED,'Completed'),(FAILED,'Failed')]

user        = models.ForeignKey(settings.AUTH_USER_MODEL, on_delete=models.CASCADE)

quantity    = models.PositiveIntegerField(default=1)

amount      = models.DecimalField(max_digits=8, decimal_places=2)

currency    = models.CharField(max_length=3, default='USD')

status      = models.CharField(max_length=20, choices=STATUS_CHOICES, default=PENDING)

reference   = models.CharField(max_length=100, blank=True)

created_at  = models.DateTimeField(auto_now_add=True)

completed_at = models.DateTimeField(null=True, blank=True)

def fulfil(self):

"""Create tokens after successful payment confirmation."""

from django.utils import timezone

for _ in range(self.quantity):

Token.objects.create(user=self.user, source='purchase')

self.status       = self.COMPLETED

self.completed_at = timezone.now()

self.save()

## 17.2 Token Pricing Reference

| Package | Tokens | Duration | Price (USD) |
| --- | --- | --- | --- |
| Starter | 1 Token | 5 hours | $0.50 |
| Daily | 3 Tokens | 15 hours | $1.25 |
| Weekly | 10 Tokens | 50 hours | $3.50 |
| Monthly | 30 Tokens | 150 hours | $9.00 |

# 18. Media Server Integration

Jellyfin (video) and Navidrome (music) run on the LAN and are accessible only after a token is activated. The portal acts as the gateway.

## 18.1 Token Activation — Media Server Access

When a user activates a token, their client IP is added to the nftables allowed_ips set. The media servers (running on the LAN, e.g., 192.168.1.10) become reachable from that IP for the token duration.

# In dnsmasq.conf — add specific resolution for media servers

# so clients can reach them by hostname after token activation:

# Assuming Jellyfin runs on 192.168.1.10:8096

address=/jellyfin.webmart/192.168.1.10

# Assuming Navidrome runs on 192.168.1.10:4533

address=/music.webmart/192.168.1.10

## 18.2 nftables Rule for Media Server Access

# Add to the FORWARD chain in /etc/nftables.conf

# Allow authenticated clients to reach specific media ports only

# Jellyfin HTTP

ip saddr @allowed_ips ip daddr 192.168.1.10 tcp dport 8096 accept

# Navidrome

ip saddr @allowed_ips ip daddr 192.168.1.10 tcp dport 4533 accept

## 18.3 Django Dashboard — Media Server Links

# In portal/views.py dashboard context, add media server URLs:

MEDIA_SERVERS = [

{'name': 'Jellyfin',   'url': 'http://jellyfin.webmart:8096', 'icon': 'film'},

{'name': 'Navidrome',  'url': 'http://music.webmart:4533',    'icon': 'music'},

]

# Pass to template only if user has an active token:

context['media_servers'] = MEDIA_SERVERS if user_has_active_token else []

# 19. Scheduled Tasks — Crontab

# Edit crontab for root (needed for nft commands):

crontab -e

# Add these entries:

# Poll bandwidth every 5 minutes

*/5 * * * * /opt/webmart/venv/bin/python /opt/webmart/manage.py poll_bandwidth

# Expire tokens and close sessions every minute

* * * * * /opt/webmart/venv/bin/python /opt/webmart/manage.py expire_tokens

# Daily cleanup of old lease data

0 3 * * * /opt/webmart/venv/bin/python /opt/webmart/manage.py cleanup_sessions

# Rotate logs weekly

0 0 * * 0 logrotate /etc/logrotate.d/webmart

## 19.1 Token Expiry Management Command — tokens/management/commands/expire_tokens.py

import subprocess

from django.core.management.base import BaseCommand

from django.utils import timezone

from tokens.models import Token

class Command(BaseCommand):

help = 'Mark expired tokens and remove IPs from nftables'

def handle(self, *args, **kwargs):

expired = Token.objects.filter(

used=True,

expired=False,

expires_at__lt=timezone.now()

)

for token in expired:

token.expired = True

token.save()

# Close associated analytics session

try:

session = token.session

if not session.end_time:

session.end_time = timezone.now()

session.save()

ip = session.ip_address

# Remove from nftables (nft auto-times-out, this is belt-and-braces)

subprocess.run(['nft','delete','element','ip','nat','allowed_ips',

f'{{ {ip} }}'], check=False)

subprocess.run(['nft','delete','element','ip','filter','allowed_ips',

f'{{ {ip} }}'], check=False)

except Exception:

pass

self.stdout.write(f'Expired {expired.count()} tokens.')

# 20. Logging & Log Rotation

cat > /etc/logrotate.d/webmart << 'EOF'

/var/log/webmart/*.log {

weekly

rotate 8

compress

missingok

notifempty

postrotate

supervisorctl restart webmart_gunicorn > /dev/null 2>&1 || true

endscript

}

/var/log/dnsmasq.log {

daily

rotate 14

compress

missingok

postrotate

systemctl kill -s HUP dnsmasq > /dev/null 2>&1 || true

endscript

}

EOF

# 21. Security Hardening

## 21.1 Django Security Settings

# Add to config/settings.py

SECURE_BROWSER_XSS_FILTER    = True

X_FRAME_OPTIONS              = 'DENY'

SECURE_CONTENT_TYPE_NOSNIFF  = True

SESSION_COOKIE_HTTPONLY      = True

CSRF_COOKIE_HTTPONLY         = True

# Rate-limit login attempts (install django-ratelimit)

# pip install django-ratelimit

# Password validation

AUTH_PASSWORD_VALIDATORS = [

{'NAME': 'django.contrib.auth.password_validation.MinimumLengthValidator',

'OPTIONS': {'min_length': 8}},

{'NAME': 'django.contrib.auth.password_validation.CommonPasswordValidator'},

{'NAME': 'django.contrib.auth.password_validation.NumericPasswordValidator'},

]

## 21.2 System Hardening

- Run Gunicorn as www-data, not root

- PostgreSQL password authentication enforced (pg_hba.conf)

- SSH on eth0 only — disabled on wlan0 via nftables INPUT chain

- Admin path changed: /admin/ → /webmart-control-<random>/

- ALLOWED_HOSTS strictly set to 192.168.10.1 and portal hostname

- Django DEBUG=False in production — never expose traceback to clients

## 21.3 nftables Additional Protection

# Add to nftables.conf — DoS mitigation on the captive portal

table ip filter {

chain INPUT {

# Rate-limit new connections to portal HTTP

iif wlan0 tcp dport 80 ct state new limit rate 20/minute accept

iif wlan0 tcp dport 80 ct state new drop

}

}

# 22. Testing & Verification

## 22.1 Network Layer Tests

# From another machine / phone connected to 'Webmart Guest':

# 1. Verify DHCP assignment

ip addr   # Should show 192.168.10.x

# 2. Verify DNS redirect

nslookup google.com 192.168.10.1

# Should return: 192.168.10.1 (portal IP)

# 3. Verify HTTP redirect

curl -v http://google.com

# Should receive redirect to portal

# 4. Check captive portal detection

curl http://connectivitycheck.gstatic.com/generate_204

# Should redirect to portal

## 22.2 Application Tests

# On the container:

cd /opt/webmart && source venv/bin/activate

# Run Django tests

python manage.py test accounts tokens devices analytics

# Check gunicorn is running

supervisorctl status

# Check nginx is passing requests

curl -H 'Host: 192.168.10.1' http://localhost/

## 22.3 Token Flow End-to-End Test

- Connect phone to 'Webmart Guest' SSID

- Browser auto-opens captive portal (or manually go to http://192.168.10.1)

- Register with email, phone, and date of birth

- Confirm 5 tokens appear in dashboard

- Click 'Activate' on one token

- Verify internet access works (e.g., browse a site)

- Verify nftables entry: nft list set ip nat allowed_ips

- After 5 hours, verify token marked expired and access removed

# 23. Service Startup Sequence & Summary

The following is the correct order in which services must start. This is handled automatically by systemd dependencies after initial configuration.

| # | Service | Command | Depends On |
| --- | --- | --- | --- |
| 1 | Networking | systemctl start networking | — |
| 2 | hostapd | systemctl start hostapd | wlan0 up |
| 3 | dnsmasq | systemctl start dnsmasq | wlan0 static IP |
| 4 | nftables | systemctl start nftables | networking |
| 5 | PostgreSQL | systemctl start postgresql | — |
| 6 | Redis | systemctl start redis | — |
| 7 | Supervisor | systemctl start supervisor | postgres, redis |
| 8 | Nginx | systemctl start nginx | supervisor (gunicorn) |

# Enable all services to auto-start on boot:

systemctl enable hostapd dnsmasq nftables postgresql redis-server supervisor nginx

# Verify all are running:

systemctl is-active hostapd dnsmasq nftables postgresql redis-server supervisor nginx

# 24. Troubleshooting Reference

| Symptom | Likely Cause | Resolution |
| --- | --- | --- |
| SSID not visible | hostapd failed to start | Check: journalctl -u hostapd; verify wlan0 supports AP mode |
| No IP from DHCP | dnsmasq not listening on wlan0 | Check: ss -ulnp ¦ grep 67; restart dnsmasq |
| Portal not redirecting | nftables rules not loaded | Run: nft list ruleset; re-apply /etc/nftables.conf |
| 502 Bad Gateway | Gunicorn not running | Run: supervisorctl restart webmart_gunicorn |
| No MAC in DB | ARP table empty | Ensure client sent traffic; check arp -n from container |
| Token not opening access | nft command failed / sudo | Ensure Django runs as root or has sudo access to nft |
| Bandwidth shows 0 | nft JSON parsing not implemented | Implement full nft -j parser in poll_bandwidth command |

# 25. Final Directory Structure

/opt/webmart/

├── config/

│   ├── settings.py

│   ├── urls.py

│   └── wsgi.py

├── accounts/

│   ├── models.py       # WebmartUser

│   ├── views.py        # register, login

│   ├── forms.py

│   ├── urls.py

│   └── admin.py

├── tokens/

│   ├── models.py       # Token, TokenPurchase

│   ├── views.py

│   ├── urls.py

│   └── management/commands/expire_tokens.py

├── devices/

│   ├── models.py       # Device

│   ├── middleware.py   # DeviceFingerprintMiddleware

│   └── admin.py

├── analytics/

│   ├── models.py       # Session

│   ├── admin.py

│   └── management/commands/poll_bandwidth.py

├── portal/

│   ├── views.py        # dashboard, activate_token

│   ├── urls.py

│   └── templates/

│       └── portal/

│           └── dashboard.html

├── templates/

│   ├── base.html

│   └── accounts/

│       ├── login.html

│       └── register.html

├── staticfiles/        # collected by collectstatic

├── manage.py

├── .env

└── requirements.txt

# 26. requirements.txt

Django>=4.2,<5.0

psycopg2-binary>=2.9

django-crispy-forms>=2.0

crispy-bootstrap5>=0.7

python-decouple>=3.8

gunicorn>=21.0

whitenoise>=6.6

Pillow>=10.0

celery>=5.3

redis>=5.0

django-ratelimit>=4.0

djangorestframework>=3.14

# Appendix — Quick Command Reference

| Action | Command |
| --- | --- |
| Restart entire stack | supervisorctl restart all && systemctl restart nginx |
| View live portal logs | tail -f /var/log/webmart/gunicorn_access.log |
| List connected clients | arp -n ¦ grep 192.168.10 |
| List authenticated IPs | nft list set ip nat allowed_ips |
| Manually allow IP | nft add element ip nat allowed_ips { 192.168.10.x timeout 18000s } |
| Django shell | cd /opt/webmart && source venv/bin/activate && python manage.py shell |
| DB backup | pg_dump webmart_db > /backup/webmart_$(date +%F).sql |
| View DHCP leases | cat /var/lib/misc/dnsmasq.leases |
| Check AP clients | iw dev wlan0 station dump |

NOTE: This document covers the core architecture. For production, add HTTPS via Let's Encrypt (requires DNS pointing to your WAN IP), a payment gateway for token purchases (Paynow, Stripe), and optionally a branded Bootstrap 5 frontend theme for the portal registration page.